PRDE-osiatd-2019-005-HYPER CONVERGED DATA CENTER AND backup services RFP

FORM 2.A – HYPER CONVERGED DATA CENTER SOLUTION GENERAL CHARACTERISTICS

| hyper converged data center SOLUTION General Characteristics |  |  |  |
| --- | --- | --- | --- |
| General Descriptions | Mandatory for Points | Complies (Yes/No) | Notes |
| Technical Specifications for the Hyper-Converged System will reference the following subcomponents: | Yes |  |  |
| - Hyper-Converged Nodes |  |  |  |
| - Virtualized Servers |  |  |  |
| - Virtualized Storage |  |  |  |
| - Software Defined Data Center Management System |  |  |  |
| Hyper-Converged system must be delivered in a complete and unified way: Hardware nodes, virtualized servers, virtualized Network and virtualized storage and management system will be supported from a unified management console, dedicated floor to ceiling caged closed containment for PRDE Hyperconverged Data Center equipment, the server cage entrance & exit should be made accessible using a biometric/keypad access system with two factor authentications (i.e.: fingerprint sensor/keyboard/keypad + access card control system) in addition to a physical lock and access log system. | Yes |  |  |
| Product certified and integrated by the manufacturer as a Hyper-Converged System in all its components: Hyper-Converged Nodes, Virtualized Servers, Virtualized Storage and Virtualized Network | Yes |  |  |
| Complete software stack for the Hyper-Converged System, which includes Virtualized Servers, Virtualized Storage, Virtualized Network and Management System. All these systems must be included by the provider of the solution. | Yes |  |  |
| Components offered must all be Brand-new, not re-manufactured, nor repaired, or refurbished components. | Yes |  |  |
| Software updates, firmware, patches, fixes must be certified and delivered by the Proposer and includes all components: Virtualized Servers, Virtualized Storage, virtualized network and Management System. | Yes |  |  |
| Proposer must offer and certify a warranty support plan that includes direct phone support from a unified support center where all problems can be addressed for the virtualization technology, servers, storage, and networking components. | Yes |  |  |
| Proposer must provide a warranty that the hardware provided pursuant to the contract will function as designed for a period of no less than three (3) years or better from the date of system acceptance. The warranty shall require the Proposer correct, at its own expense, the setup, configuration, customizations or modifications so that it functions according to the PRDE’s requirements. | Yes |  |  |
| Proposer must specify any equipment not covered by warranty and/or any other deliverables exclusion from warranty work. | Yes |  |  |
| Proposer must provide evidence of local availability for replacement parts (within Puerto Rico). This includes spare parts to cover SLAs and required support times for hardware failure. | Yes |  |  |
| System must count with application(s) support that reports system health automatically to the manufacturer. | Yes |  |  |
| Hardware support service must provide the option of retaining failed hard drives, ensuring that the Department of Education is not exposed to data loss/theft. | Yes |  |  |

FORM 2.B – HYPER CONVERGED NODES

| Hyper-Converged Nodes |  |  |  |
| --- | --- | --- | --- |
| General Description | Mandatory for Points | Complies (Yes/No) | Notes |
| Hyper-Converged system must accommodate from 3 to 64 nodes in increments of one node at a time. | Yes |  |  |
| System must be able to accommodate nodes with different characteristics such as the following: | Yes |  |  |
| - High capacity storage nodes, each node 2 Rack Units, |  |  |  |
| Specific Hyper-Converged Nodes |  |  |  |
| Hyper-Converged System must be able to scale to a maximum of 64 nodes per cluster. | Yes |  |  |
| Each node must have the following network ports: | Yes |  |  |
|  Two (2) 10Gb Ethernet ports  Two (2) 1Gb Ethernet ports |  |  |  |
|  One (1) 10/100Mb Ethernet Management port |  |  |  |
| Each node must have the following computing capacity: | Yes |  |  |
|  Sockets: 2 |  |  |  |
|  Cores: 36 |  |  |  |
|  RAM: 512GB |  |  |  |
| Each node must have the following storage capacity: | Yes |  |  |
|  Cache Drives: minimum of 1 x 400GB |  |  |  |
|  Capacity Drives: minimum 3.84 TB SSD |  |  |  |

FORM 2.C – VIRTUALIZED SERVERS (HYPERVISOR)

| Virtualized servers (Hypervisor) |  |  |  |
| --- | --- | --- | --- |
| General Description | Mandatory for Points | Complies (Yes/No) | Notes |
| Hyper-Converged system must come installed with Hypervisor Management Software. | Yes |  |  |
| Hyper-converged system must provide integrated /engagement support of the virtualized server layer | Yes |  |  |
| Hypervisor must include distributed virtual switches to manage all configurations as a single group. | Yes |  |  |
| Hypervisor must have an integrated log to record hardware and software events. | Yes |  |  |
| Hypervisor must include automated High-Availability features, automatic resource distribution and online storage migrations. | Yes |  |  |

FORM 2.D – VIRTUALIZED STORAGE

| Virtualized Storage |  |  |  |
| --- | --- | --- | --- |
| General Description | Mandatory for Points | Complies (Yes/No) | Notes |
| Hyper-converged system must include preloaded Hypervisor Storage System Software. | Yes |  |  |
| Virtualized storage layer must run on the Hypervisor’s kernel | Yes |  |  |
| Virtualized storage must be configured as AFA (All Flash Array) SSD and support future mixed environments. | Yes |  |  |
| SSD drives must be high performance devices providing cache-writing functionality. |  |  |  |
| Virtualized storage management must be part of the Hypervisor Management console. | Yes |  |  |
| Virtualized Storage system must have policies to manage down to the level of the virtual drives: | Yes |  |  |
|  Performance |  |  |  |
|  Protection Levels |  |  |  |
|  Quality of Service |  |  |  |

FORM 2.E – VIRTUAL MACHINE REPLICATION

| Virtual Machine Replication |  |  |  |
| --- | --- | --- | --- |
| General Description | Mandatory for Points | Complies (Yes/No) | Notes |
| Hyper-converged system must provide Virtual Machine Replication functionality to remote systems with the same hypervisor. | Yes |  |  |

FORM 2.F – CLOUD HIERARCHICAL STORAGE MANAGEMENT

| Cloud Hierarchical Storage Management |  |  |  |
| --- | --- | --- | --- |
| General Description | Mandatory for Points | Complies (Yes/No) | Notes |
| Solution should have the ability to replicate hierarchical storage to external cloud services. | Yes |  |  |

FORM 2.G – NETWORKING EQUIPMENT REQUIREMENTS

|  | networking equipment requirements |  |  |  |
| --- | --- | --- | --- | --- |
| General Description |  | Mandatory for Points | Complies (Yes/No) | Notes |
| Capable of IPsec VPN tunnels and able to connect management systems using the following algorithms: AES, AES256, Triple DES, AES-GCM, 2-DH y DH-5 |  | Yes |  |  |
| L2VPN tunneling creation |  | Yes |  |  |
| Remote Access VPN to support SSL VPN connections. |  | Yes |  |  |
| Capable of handling VXLAN tunnel extensions used by virtual machines for Data Center access from geographically dispersed environments. |  | Yes |  |  |
| Packet routing in VXLAN networks for optimizing traffic and bandwidth utilization. |  | Yes |  |  |
| Capable of supporting same networking access rules from different locations. |  | Yes |  |  |
| External routing only to the virtualized environment. |  | Yes |  |  |
| Single Management Console supporting a minimum of 8 remote virtual networks. |  | Yes |  |  |
| Capable of Virtual Extensible LAN (VXLAN) tunnels, routers and firewalls for Data Centers and able to restrict to a single location. |  | Yes |  |  |
| Capable of connecting Virtual Extensible LAN (VXLAN) and 802.1Q VLAN segments using physical devices from at least 3 different certified equipment manufacturers. |  | Yes |  |  |

FORM 2.H –NETWORK INSIGHT GENERAL CHARACTERISTICS

| Network Insight General Characteristics |  |  |  |
| --- | --- | --- | --- |
| General Description | Mandatory for Points | Complies (Yes/No) | Notes |
| Provide planning capabilities for security visibility and traffic analysis, troubleshooting, auditing and networking traffic compliance of environments that support physical or virtualized topologies. | Yes |  |  |
| Provide a web-based graphical interface that can easily monitor large data flows with the capacity to drill-down into each individual element. | Yes |  |  |
| Provide 360 degree visibility between physical and virtual networks. | Yes |  |  |
| Provide East-West, North-South traffic analysis, and between Hosts at the L2 or L3 layers. | Yes |  |  |
| Provide internal and external traffic visibility between layers or intra layers including VLAN and Virtual Extensible LAN (VXLAN) segments. | Yes |  |  |
| Capability to model security groups. | Yes |  |  |
| Capability to monitor IPFIX flows and group them in VLAN or Virtual Extensible LAN (VXLAN) segments within vCenter including micro segmentation. | Yes |  |  |
| Capability to suggest security policies that can be exported such as micro-segmentation policies within the environment. | Yes |  |  |
| Include a search interface that will allow basic and complex searches. | Yes |  |  |
| Capacity to identify potential connectivity issues. | Yes |  |  |
| Provide analytics of the virtualized infrastructure that includes storage, servers, computers, networking, and security components. Able to analyze both cloud and physical connections and workloads via Virtual Extensible LAN (VXLAN) or VLAN. | Yes |  |  |
| Able to collect configuration information and routing tables from networking devices using SSH or SNMP from any network device including brands such as Cisco, HP, Dell, Arista, Brocade or Juniper. | Yes |  |  |
| Capacity to collect, analyze and process IPFIX information through VLAN/VXLAN, routing, or distributed firewall. | Yes |  |  |
| Able to create alerts when certain defined parameters are broken. | Yes |  |  |
| Capability to retain historic networking workflows for approximately 30 days. | Yes |  |  |
| Able to collect IPFIX information from the virtual network switches. | Yes |  |  |