FORM 8

SERVICE FUNCTIONALITY COMPLIANCE

| MINIMUM FUNCTIONALITY SPECIFICATION | MEETS | EXCEEDS | DOES NOT MEET | NOTES/EXCEPTIONS |
| --- | --- | --- | --- | --- |
| A. Transport Infrastructure: |  |  |  |  |
| 1. Proposer must be capable of delivering a minimum bandwidth or committed information rate (CIR) of 20G. Bandwidth can be adjusted per school density and profile (elementary vs high school) either up and down. On an MPLS, the bandwidth and related cost can also be adjusted. Proposers must present an innovative and cost effective design and solution. |  |  |  |  |
| 2. The Department would like the ability to increase and/or vary bandwidth in the future to meet demand. Proposals may include rates for higher bandwidth connections and should describe the Proposer’s ability to scale the proposed solution to meet greater bandwidth demands. |  |  |  |  |
| 3. The Proposer is solely responsible for obtaining and paying for related costs for any permits. |  |  |  |  |
| 4. The Proposer is responsible for creating a fiber service entrance into each building and will terminate the connection at the MDF. |  |  |  |  |
| 5. Proposers must provide one router per Region Office that has the capability to run from 10G up to more than 20G transport and Internet speed (minimum request). The router must have one (1) 24 GbE LAN ports and more than 6 GbE WAN ports (fiber). |  |  |  |  |
| 6. The Proposer must provide a one router per school that has the capability to run at least from 750Mbps to 1G transport speed (minimum request). The router must have at least four GbE LAN ports and at least two GbE WAN ports. |  |  |  |  |
| a. One for connect to Regional Office – WAN main connection (Fiber). |  |  |  |  |
| b. One for connect to secondary network – WAN redundancy. |  |  |  |  |
| c. One for school’s LAN –voice and data. |  |  |  |  |
| d. One for school’s BYOD – WiFi. |  |  |  |  |
| 7. The Department also requests at least seven (7) WAN network segments for transport that should be configured in the router in each of the Region Offices. Each school will belong to a one network segment according to the educational Region to which the school belongs. |  |  |  |  |
| 8. The Department would like to retain the same type of infrastructure devices that are currently in the Department’s Data Center, but on smaller scale, based on the number of schools per Region. |  |  |  |  |
| 9. There should be at least two (2) route traffic options from every Region Office to the Data Center. |  |  |  |  |
| 10. Designs must present some network resilience capabilities. |  |  |  |  |
| 11. The Department intends to continue using the current school Data Center infrastructure. Given that preference, Proposers will need to analyze the configuration to reroute the traffic and link to each Region Office. In addition, the infrastructure will use for a passive backup internet circuit. |  |  |  |  |
| 12. Proposers must provide a complete network solution that includes, but is not limited to: |  |  |  |  |
| a. Security/Firewall (UTM package). |  |  |  |  |
| b. Security/Web content filtering (Comply with CIPA). |  |  |  |  |
| c. Dashboard solution for Reports, data analyzer, etc. |  |  |  |  |
| d. Broadband business continuity: |  |  |  |  |
| Fiber provider |  |  |  |  |
| Wireless provider |  |  |  |  |
| e. Intranet circuit to Department’s Data Center. |  |  |  |  |
| 13. Telecommunication Room – Turnkey solution, but no limited to: |  |  |  |  |
| a. Power and grounding options. |  |  |  |  |
| b. Battery Backup Options. |  |  |  |  |
| c. Rack options. |  |  |  |  |
| d. Lighting options. |  |  |  |  |
| e. May be the s will be need construct a new Telecommunication Room. |  |  |  |  |
| f. Cooling options. |  |  |  |  |
| g. Floor options. |  |  |  |  |
| B. Internet Infrastructure: |  |  |  |  |
| As mentioned above, the Department has seven (7) Educational Region Offices (ORE) that each manage between 120 to 150 schools. The Department requests that Proposers install and configure public IP network segments for Internet on each of the seven (7) Educational Region Offices. Each network segment per Region should comply with CIPA and with Azure accounts and profile integrations. In addition, the Internet infrastructure should meet the following requirements: |  |  |  |  |
| 1. The Department would like the ability to increase and/or vary Internet speed in the future to meet demand. Proposals may include rates for higher speeds and should describe the Proposer’s ability to scale the proposed solution to meet greater Internet demands. |  |  |  |  |
| 2. The Department would like to retain the same type of infrastructure devices that are currently in the Department’s Data Center, but on smaller scale, based on the number of schools per Region. |  |  |  |  |
| 3. Two (2) Internet circuit options that provide network resilience capabilities, one (1) of which shall be configured with network load balance to the secondary circuit. This should be the configuration for each of the seven (7) Region Offices. |  |  |  |  |
| 4. The Department intends to continue using the school’s Data Center infrastructure. As a result, Proposers must analyze the configuration to reroute the traffic and link to each Region Office as a passive backup Internet circuit. |  |  |  |  |
| 5. For the main active Internet circuit, the Department requests a fiber optic installation to every one of the seven (7) Region Offices, and a static /29 public IP Address segment to be distributed between seven (7) network regions. Proposers must show that they have more than one Internet route for their internal network load balancing so the network will continue working one (1) or more regions breakdown. |  |  |  |  |
| 6. For the secondary passive Internet circuit, the Department requests a static /29 public IP Address segment to be distributed on each of the seven (7) Educational Region Offices. The proposed circuit option can be a wireless solution. |  |  |  |  |
| 7. Proposers must provide a complete solution but not limited to: |  |  |  |  |
| a. Security/Firewall (UTM package). |  |  |  |  |
| b. Security/Web content filtering (Comply with CIPA). |  |  |  |  |
| c. Dashboard solution for Reports, data analyzer, etc. |  |  |  |  |
| d. Broadband business continuity, resilience options. |  |  |  |  |
| e. Fiber provider. |  |  |  |  |
| f. Wireless provider. |  |  |  |  |
| C. Network Management and Monitoring |  |  |  |  |
| The Department requests a dashboard where the IT staff can manage the WAN in a central console and provide better resources to manage the administration, monitoring and data collection for and custom real-time network reports. The Dashboard must provide the following, at a minimum: |  |  |  |  |
| 1. Show the health of 7 network segments. |  |  |  |  |
| 2. Show the health of each school transport circuit and router. |  |  |  |  |
| 3. Show the basic configuration for every school’s routers. |  |  |  |  |
| 4. Show critical, major, minor and informational network alerts. |  |  |  |  |
| 5. Show the service tickets that are open with the Proposer. |  |  |  |  |
| 6. Show the service tickets that were closed and the resolution document. |  |  |  |  |
| 7. Show the status progress during installation, maintenance and repair of a circuits and devices. |  |  |  |  |
| 8. Give administrative users the ability to access network configuration for any school. |  |  |  |  |
| 9. Provide multiple user access levels. |  |  |  |  |
| D. Maintenance/Support |  |  |  |  |
| Proposers will be responsible for the ongoing support of the network. The services shall include, but are not limited to: |  |  |  |  |
| 1. New installations. |  |  |  |  |
| 2. Circuit repairs. |  |  |  |  |
| 3. Fiber optic cable splicing. |  |  |  |  |
| 4. Leased equipment (routers and switches) repairs and replacements. |  |  |  |  |
| 5. Equipment firmware updates, follow the Department’s Change Control and Windows Maintenance policies. |  |  |  |  |
| 6. The Department requires Proposer’s technical staff be in place at each Region Office at least three days a week per three hours for management of all relevant circuits and services, with proactive processes to detect and avoid issues before they degrade service levels. The Department will provide a physical access to the Region Office, work area and consultant ID. |  |  |  |  |
| 7. Proposers must have a help desk staffed 7x5x7,provide via a Web portal, to be used by authorized Department personnel for: |  |  |  |  |
| a. Service ordering. |  |  |  |  |
| b. Opening tickets, status updates and resolution of trouble tickets. |  |  |  |  |
| c. Handling billing inquiries and disputes. |  |  |  |  |
| d. Undertaking soft moves, adds and changes. |  |  |  |  |
| 8. Proposals shall be required to troubleshoot technical issues such as connectivity, browsing and configuration that can affect the device and network performance. |  |  |  |  |
| 9. Proposers will maintain an electronic inventory of equipment and circuits and will provide an updated copy to the Department’s SharePoint web portal and will be accessible for the Department’s IT staff. Proposers will be required to update the inventory within forty-eight (48) hours of any changes. The inventory will track the following, at a minimum: |  |  |  |  |
| a. Regional Office ID. |  |  |  |  |
| b. School name (if applicable). |  |  |  |  |
| c. School code. |  |  |  |  |
| d. Physical address of where the equipment is installed. |  |  |  |  |
| e. Make. |  |  |  |  |
| f. Model. |  |  |  |  |
| g. Serial number. |  |  |  |  |
| h. Firmware or Operating System version, including patch level. |  |  |  |  |
| i. Circuit ID. |  |  |  |  |
| E. Trouble Reporting and Problem Resolution |  |  |  |  |
| Proposers shall, prior to commencing performance under the contract, provide a call center with sufficient telephone lines and integrated with the Proposer’s system for quality review and reporting capabilities for the entire contract. Proposals shall include a sample plan for call center operations, including: |  |  |  |  |
| 1. Location(s) and Staffing levels. |  |  |  |  |
| 2. Support Tools and Resources. |  |  |  |  |
| 3. Department’s ability to access and view tickets, and to generate daily, weekly and monthly. |  |  |  |  |
| 4. Samples of available reports. |  |  |  |  |
| 5. Hours of Operation (regular and extended) 7x5x7. |  |  |  |  |
| 6. Call Center methodologies for handling and processing calls with samples of forms and computer screen shots. |  |  |  |  |
| 7. Contact response service levels for telephone calls, voice mails and emails. |  |  |  |  |
| 8. Proposer’s levels of support, as required. |  |  |  |  |
| The Proposer’s proposal should describe procedures for trouble reporting and problem resolution and define the escalation procedures for service interruptions. Include: |  |  |  |  |
| 1. Definition of “Major Outage”. |  |  |  |  |
| 2. Guaranteed response times for repair. |  |  |  |  |
| 3. Network or circuit monitoring capabilities and responsibilities. |  |  |  |  |
| 4. List of names/positions with associated contact information for escalation of repair or outage situations. |  |  |  |  |
| Proposals should describe how the Department’s IT staff will be notified or updated on the progress of the repairs. Is there a dedicated repair group that will support the Department’s IT? If so, describe and provide contact information. |  |  |  |  |
| Proposals should also describe how Proposers will integrate Proposers’ trouble-ticketing system with the Department’s system. |  |  |  |  |
| 1. Provide detailed workflow of service processes and responses. |  |  |  |  |
| 2. Provide device and configuration technical support. |  |  |  |  |
| F. Installation and Implementation Plan: |  |  |  |  |
| Once a Proposer or Proposers are selected, the Department will request a kickoff meeting to establish an implementation, installation and configuration plan, and if needed, a transition plan. The prioritization for installations shall be as follows: |  |  |  |  |
| 1. Region Office configuration and installation devices. |  |  |  |  |
| 2. School network migration as a parallel service that will avoid or reduce to a minimal service interruptions. All of the tasks will be make without disconnecting any school or shutting down the main WAN/Internet connection at the Department’s Data Center. |  |  |  |  |
| Proposer must provide, but not limited to: |  |  |  |  |
| 1. Provide a project manager who will oversee the overall deployment, troubleshoot issues, handle logistics, and serve as liaison between the Proposer and the Department. |  |  |  |  |
| 2. Provide onsite technical support for assessing and troubleshooting reported device issues across multiple locations. |  |  |  |  |
| G. Transition Plan for Proposed WAN and Internet Services |  |  |  |  |
| For Proposers other than the current provider, provide a transition plan to migrate services from the current provider. The plan should include required actions and a target completion date. Activities during the transition process must not disrupt day-to-day educational or business functions and must be included at no additional cost to the Department. This plan will be approved by the Department in advance of the start of services and must address the following: |  |  |  |  |
| 1. Operational impact analysis |  |  |  |  |
| 2. Responsibilities matrix for Proposer, current service provider and the Department |  |  |  |  |
| 3. Formal project plan, including dependencies and contingencies |  |  |  |  |
| 4. Estimated timeframes of transition and transition start date |  |  |  |  |
| 5. Documented and approved processes, procedures and standards |  |  |  |  |
| 6. Escalation matrix with names and valid contact information |  |  |  |  |